SLC Feedback control system

The 'old' situation for SLC (1990):

- Slow control loops (about 12)

- Run on a central VAX/VMS

- Measure & correct every minute

- Easy to implement

- Database driven

- Use hardware that is used for "trims"

- Fast control loops (only 2, more wanted)

- Fast local cpu with RTOS

- Measure & correct at 120 Hz

- Processors have direct hardware access

- Dedicated software (maintenance, arrrgh !)

- Difficult to change/add loops

- 1 loop = 1 task = 1 processor = 1 actuator set

- very successful !

Would prefer :

- more unified approach

- easier implementation/maintenance

- lower costs/loop

- flexibility

SLC/PEPII  Feedback control system

The 'new' system for SLC/PEPII  (1991):

- Use a single digital control theory for design of all controllers (i.e. state space formalism)

- Distinguish 3 tasks :

- Measurements

- Compute system states

- Set actuators

- Forget about individual loops/gains :

- One "loop" has many sensors & actuators (!)

- Compute "states"

- Set actuators according to "states"

- Splitting up/design is left to users

- Not all sensors/actuators on 1 machine so :

- Point to point rt links between cpu's

(to be replaced by rt network later on)

- Control tasks distributed over many cpu's

- Generic, database driven software

- Speed loop : 120 Hz

Communicate with VAX : 1 Hz

State space formalism (1)

i) The plant is in a given "state"

e.g (x,x',y,y',E) in the case of an orbit correction

ii) From the "state vector " your predict the next state

using the knowledge of the plant dynamics

State space formalism (2)

Motivation

- Accelerator has no dynamics, the plant has, i.a.w modern feedback design methods do not work well

- Streamlined software & computations

- Better adapted to slc and is standard technique

- Database driven

- Only matrix & vector multiplications

- Changes /add stuff is simple

- Better response time (you know about plant !)

- Handle dead sensors/ bad pickup is simple

- Can use adaptive controller for example later

State space formalism (3)

- Can all our feedback control problems be considered linear ?

- Performance over a large dynamic range ?

- Do we know are "plant" equations well enough ?

- …

MULTIPLE CHOICE

Q :

What will determine the reference settings for the sextupole spool pieces in the LHC ?

A:

- Algorithm in the BI crate

- PC converter software (server/dico)

- Real time knobs (control room)

- Reference magnets in SM-18

- Database

- Central RT control system

- All of the above

- None of the above

State-Space formalism (3)

= describe the dynamics of a linear system in a single matrix notation

x(t) = Ax(t) + Bu(t)

y(t) = Cx(t) + Du(t)

x = vector of states (position, angle or energy of the beam at a given location)

y = vector of outputs (BPM readings)

u = vector of inputs (requested current for a magnet)